BIOBUZZ 2026-27 – Team 37102

# Shooting Guide – one spot, one speed

The robot’s flywheel and windmill are now consistent (robot logs 30 Oct: every ball left at 1200–1260 t/s). Misses come from distance and aim: one speed only lands from one distance. So we tune one shooting spot and always shoot from it.

0.  Before you start

- Practice HIVE with the AprilTags on the bottom of the CELLs (IDs 30–45, printed for practice from the FIRST field resources page). Without them the camera has nothing to see.

- In TeleOp INIT choose the alliance that matches the HIVE you shoot at: B / Circle = RED, A / Cross = BLUE. The camera only looks for that alliance’s CELLs.

- Fresh battery (12.5 V or more), 4 balls, tape measure, coloured tape.

1.  Pick ONE shooting spot and tape it

- Choose a spot in front of your alliance’s HIVE that drivers can reach quickly and that is not in a lane the other alliance drives through.

- Pick a comfortable distance – start around 4–6 ft from the floor under the CELL. The camera sees the tags better when not too far.

- Put tape on the floor where the front edge and both sides of the robot go (an “L” at two corners works well), robot square to the HIVE.

- Write down where the spot is using things that are the same on every field (tile seams count from the walls, distance from the HIVE frame):

|  | Our spot (fill in) |
| --- | --- |
| Alliance / HIVE side |  |
| Tiles from our alliance wall (count seams) |  |
| Tiles from the audience / rear wall |  |
| Floor distance, camera lens → point under the CELL tags | in |
| Robot heading (which wall is behind the launcher) |  |

2.  Find the speed: BioBuzz Shot Log

Robot on the tape, square to the HIVE. Select BioBuzz Shot Log, press INIT, then PLAY. D-pad = the + shaped arrows on the left of the controller; in Shot Log it changes the speed being tested.

| Button | What it does |
| --- | --- |
| Y / Triangle | Flywheel ON / OFF (starts at 1250 t/s) |
| D-pad UP / DOWN | Speed +50 / −50 t/s (big steps) |
| D-pad RIGHT / LEFT | Speed +10 / −10 t/s (fine steps) |
| RB (right bumper) | Fire ONE ball (only when the flywheel is at speed) |
| Then A / Cross | Ball went IN the CELL |
| Then B / Circle | MISS |
| Then Back / Share | Discard (no ball came out) |
| Triggers | Intake in / out to reload |

- Flywheel ON (Y). Fire 10 balls at 1250, marking each one IN or MISS.

- Mostly SHORT (hits the front / falls before the CELL) → D-pad UP (+50). Mostly LONG (over the back) → D-pad DOWN (−50).

- When it is close, use RIGHT / LEFT (+/−10) and repeat 10 balls.

- The Driver Hub shows the hit rate for each speed. Keep the speed with 8 or more of 10 IN.

CHECK:  Do not move the robot between shots – re-check it is on the tape after reloading. Left/right misses are aim, not speed.

| Speed (t/s) | IN | MISS short | MISS long | MISS left/right |
| --- | --- | --- | --- | --- |
| 1250 |  |  |  |  |

DONE WHEN:  One speed gives 8/10 or better from the tape. Final speed: ________ t/s

3.  Put the speed in the robot

- Tell the programmer (or Copilot) the speed. In BioBuzzConstants.java:

- LAUNCHER_NEAR_VELOCITY = your speed (TeleOp default, D-pad DOWN = NEAR).

- AUTO_LAUNCH_VELOCITY = the same, only if AUTO shoots from about the same distance.

- Rebuild, deploy, then fire 5 balls in TeleOp from the tape to confirm.

4.  Camera and AIM check: BioBuzz Vision Test

- Robot on the tape. Select BioBuzz Vision Test, INIT, PLAY. B / Circle switches alliance – pick the HIVE you face.

- The screen should show your CELL with 2–4 tags and a bearing near 0. Hold X / Square: the robot turns until the bearing settles.

- In TeleOp the line “HIVE aim” now always shows what the camera sees (cell, tags, bearing, range) – even when you are not pressing X.

CHECK:  “no RED CELL tags in view (any tags: 3)” means tags are seen but they belong to the other alliance’s CELL – fix the alliance in INIT. “any tags: 0” means the camera sees no tags at all: check the practice tags, the camera tilt (30° up) and that nothing blocks the view.

DONE WHEN:  Your CELL shows with 2+ tags from the tape, and holding X turns the robot onto it.

5.  DRIVER RULE (match play)

1. DRIVE TO THE SPOT – same place every time. 2. SQUARE UP to the HIVE, launcher facing the CELL. 3. STOP – sticks released, robot still. 4. HOLD X (aim) until the bearing settles – if tags are seen. 5. HOLD RB – one ball every ~0.5 s while held; release to stop. Do NOT shoot from anywhere else. Do NOT shoot while moving.

Tip: press Y (pre-spin) while driving to the spot, so the flywheel is already at speed when you stop.

TeleOp button card

D-pad = the + shaped arrow buttons on the LEFT side of the controller. In TeleOp it only changes the SHOT SPEED (how hard the ball is thrown). It does not drive or aim.

| D-pad (TeleOp) | What it does | Use it when |
| --- | --- | --- |
| UP | FAR preset: 1450 t/s, trim reset to 0 | shooting from far away |
| DOWN | NEAR preset: 1250 t/s (or the speed you tuned), trim reset to 0 | shooting from our taped spot – normal choice |
| RIGHT | +25 t/s (each press) | balls land SHORT (in front of / below the CELL) |
| LEFT | −25 t/s (each press) | balls go LONG (over / behind the CELL) |

The Driver Hub line “Shot speed” always shows it, e.g. NEAR 1250 +25 = 1275 t/s. Pressing UP or DOWN clears the +/− trim. The speed stays until you change it; it is used the next time you press RB or Y.

CHECK:  D-pad in other programs is different: BioBuzz Shot Log = speed ±50 (UP/DOWN) and ±10 (RIGHT/LEFT); BioBuzz Calibration and Arm Setup = arm position; AUTO INIT = start delay.

| Button | Action |
| --- | --- |
| Left stick / right stick X | Drive + strafe / turn (intake side = FRONT) |
| LB (hold) | Precision (slow) drive |
| RT / LT | Intake IN / OUT (half LT = flower back-out) |
| RB (hold) | FIRE: spins up and feeds one ball at a time |
| X / Square (hold) | HIVE auto-aim (camera turns the robot) |
| Y / Triangle | Flywheel pre-spin ON / OFF |
| D-pad UP / DOWN | Shot speed FAR (1450) / NEAR (1250) – see table above |
| D-pad RIGHT / LEFT | Shot speed +25 / −25 per press |
| A / Cross | Flower arm DOWN / UP (only when stopped) |
| B / Circle (hold) | Unjam: windmill slowly backwards |
| INIT: B / A, Back | Alliance RED / BLUE; diagnostics log on/off |