BIOBUZZ 2026-27 – goBILDA StarterBot

# Wiring Sheet – which cable goes where

Left / right / front / back are seen from BEHIND the robot, looking toward the intake. FRONT = intake + flower arm. BACK = launcher + camera.

Simple rule: everything on the robot’s LEFT goes to the CONTROL HUB, everything on the RIGHT goes to the EXPANSION HUB. The launcher (middle) goes to the Control Hub.

# CONTROL HUB (left side)

| Port | Device | Encoder cable | Config type | Name (exact) | Done |
| --- | --- | --- | --- | --- | --- |
| Motor 0 | Front-left drive wheel | Encoder 0 | goBILDA 5202/3/4 | left_front_drive | ☐ |
| Motor 1 | Back-left drive wheel | Encoder 1 | goBILDA 5202/3/4 | left_back_drive | ☐ |
| Motor 2 | Launcher / flywheel motor (middle, 1:1) | Encoder 2 | goBILDA 5202/3/4 | launcher | ☐ |
| Motor 3 | SPARE – leave empty | – | – | – | ☐ |
| Servo 0 | Left corner intake wheel servo | – | Continuous Rotation Servo | left_intake_servo | ☐ |
| USB 2.0 | Logitech C270 webcam | – | Webcam | Webcam 1 | ☐ |
| (built in) | IMU inside the Control Hub – nothing to plug | – | REV internal IMU | imu | ☐ |
| I2C 1 | goBILDA Pinpoint (OPTIONAL, AUTO only) – pods into its X / Y ports | – | goBILDA Pinpoint Odometry Computer | pinpoint | ☐ |

# EXPANSION HUB (right side)

| Port | Device | Encoder cable | Config type | Name (exact) | Done |
| --- | --- | --- | --- | --- | --- |
| Motor 0 | Front-right drive wheel | Encoder 0 | goBILDA 5202/3/4 | right_front_drive | ☐ |
| Motor 1 | Back-right drive wheel | Encoder 1 | goBILDA 5202/3/4 | right_back_drive | ☐ |
| Motor 2 | Intake roller motor | NOT needed | goBILDA 5202/3/4 | intake | ☐ |
| Motor 3 | Windmill / indexer motor (435 RPM) | Encoder 3 | goBILDA 5202/3/4 | indexer | ☐ |
| Servo 0 | Right corner intake wheel servo | – | Continuous Rotation Servo | right_intake_servo | ☐ |
| Servo 1 | Flower arm servo (SERVO mode, not continuous) | – | Servo | flower_arm | ☐ |

# Power and hub link

- Battery → main power switch → Control Hub XT30 power input.

- Control Hub’s second XT30 → Expansion Hub XT30 (the Expansion Hub has no battery of its own).

- Control Hub RS485 → Expansion Hub RS485 (either RS485 port on each hub).

# Rules (check every cable)

- Encoder number = motor number. Motor in port 2 → its encoder cable in encoder port 2 on the SAME hub. Swapped encoder cables make the robot drive crazy.

- Encoders needed: all 4 drive motors, the launcher and the windmill/indexer. The intake motor needs NO encoder cable.

- Servo plugs: black (ground) wire on the pin marked – / ground on the hub. A plug turned around does nothing.

- The flower arm servo must be in SERVO (position) mode (goBILDA servo programmer) before it is installed.

- Webcam goes in the Control Hub’s USB 2.0 port (the black one), not the blue USB 3.0 port.

- Keep cables away from the wheels, the intake rollers, the flywheel, the windmill and the flower arm’s swing. Zip-tie them; leave a little slack at every moving part.

- Power the robot OFF before plugging or unplugging anything.

# When you are done

- Driver Hub: robot configuration → check every name above is typed EXACTLY (lower case, underscores). A typo stops the program at INIT.

- Robot on a box, wheels free: run BioBuzz Wiring Check. It must say WIRING OK; every FAIL line says which cable to fix.

- Then run BioBuzz System Check.

# Cable tags – cut out and tape/zip-tie one to each end of the cable

| CONTROL HUB Motor 0 left_front_drive | CONTROL HUB Motor 1 left_back_drive | CONTROL HUB Motor 2 launcher | CONTROL HUB Servo 0 left_intake_servo | CONTROL HUB USB 2.0 Webcam 1 |
| --- | --- | --- | --- | --- |
| CONTROL HUB I2C 1 pinpoint | EXPANSION HUB Motor 0 right_front_drive | EXPANSION HUB Motor 1 right_back_drive | EXPANSION HUB Motor 2 intake | EXPANSION HUB Motor 3 indexer |
| EXPANSION HUB Servo 0 right_intake_servo | EXPANSION HUB Servo 1 flower_arm | CONTROL HUB Encoder 0 left_front_drive (encoder) | CONTROL HUB Encoder 1 left_back_drive (encoder) | CONTROL HUB Encoder 2 launcher (encoder) |
| EXPANSION HUB Encoder 0 right_front_drive (encoder) | EXPANSION HUB Encoder 1 right_back_drive (encoder) | EXPANSION HUB Encoder 3 indexer (encoder) | BOTH HUBS RS485 Control Hub ↔ Expansion Hub | BOTH HUBS XT30 Expansion Hub power (from Control Hub) |